Accepted in 4th International Conference on Hybrid and Organic Photovoltaics to be celebrated in Uppsala, Sweden, 6 - 9 May 2012

# Flavylium salts as novel photosensitizers for dye-sensitized solar cells.

Giuseppe Calogero1, Alessandro Sinopoli1, Ilaria Citro1, Gaetano Di Marco1 Vesselin Petrov2, Ana M. Diniz2, A. Jorge Parola2, Fernando Pina2.

1) CNR, Istituto per i Processi Chimico-Fisici, via F.Stagno D’Alcontres 37, I-98158 Messina, Italy

2) REQUIMTE, Departamento de Química, Faculdade de Ciências e Tecnologia, Universidade Nova de Lisboa, 2829-516 Caparica, Portugal

Abstract

Seven flavylium salt dyes were employed for the first time as novel sensitizers for dye-sensitized solar cells (DSSCs). Photophysical and photoelectrochemical measurements showed that these flavylium dyes are very promising for DSSC applications [1]. Among the investigated compounds, the best performance has been obtained by a DSSC based on the novel 7-(N,N-Diethylamino) - 3’,4’-dihydroxyflavylium salt.  Our results suggests that the substitution of a hydroxylic group with an diethylamine unit (NC2H5) into the position 7 of ring A of flavylium framework expanded the p-conjugation in the dye and thus resulted in a high absorption in the visible region and is advantageous for effective electron injection from the dye into the conduction band of TiO2. In addition, the amine group, owing to its strong electron-donor ability, might play an important role in electron injection in addition to a red shift in the absorption region. This work open new possibility to develop new class of sensitizers synthetic analogues of natural anthocyanidins.

- F. Pina, M. J. Melo, C. A.T. Laia, A. Jorge Parola, J. C. Lima Chem. Soc. Rev. 2011, 41, 869.

Dye sensitized solar cell based on flavylium compound

Figura 1  (7-diethylamino-3’,4’-dihydroxyflavylium)

Figura 2 Parafilm sealed DSSC based on flayilium

Figura 3  Flavylium based photoanodes